A. TCP/IP

1. NAT is the network address translation that maps private IP addresses to one or more public IP addresses. It is used to conserve public IPv4 addresses, provides a layer of address obscuring, with one-to-one mapping, dynamic NAT. Many internal hosts share on public IP using different source ports and it is implemented on routers and firewalls on the network edge.

2. A socket is an endpoint for network communication. A port is a 16-bit number that identifies an application or a process on a host system. Port 21 is used for FTP control. Port 53 is used for the DNS of the network.

3. DNS works in this way. The App asks a local DNS resolver a request. If the resolver has a cached answer, then there is a return. If not, the resolver queries about the root servers. The resolver caches result and return the address to the client. If there was no DNS, the user must type numeric IPs for each service. Human friendly names wouldn’t be resolved, and web/e-mail addressing would either break or become cumbersome or inconvenient.

4. BootP is an older protocol that is used by diskless workstations to obtain the IP address, gateway, and boot filename and server. It uses a server-side mapping keyed by the MAC address.

5. DHCP automatically assigns the IP configuration to the hosts’ IP address, subnet mask, default gateway and DNS servers. It makes admins’ lives easier by eliminating manual static IP entry, avoids addressing conflicts and mistakes, centralizes control for address pools and reservations, allows for easier reconfiguration and supports reservations and lease times.

6. A lease is a time-limited allocation of an IP address to a client.

7. WINS resolves NetBIOS names to the IP Addresses. It is different from DNS due to the NetBIOS name resolution using Windows LAN names to dynamically register Windows clients. WINS is Microsoft specific and largely deprecated in modern DNS environments.

8.The tools that can be used to troubleshoot TCP/IP include ping, traceroute, ipconfig, netstat, nslookup, arp -a, route/iproute, telnet <host> <port>, tcdump/tshark/wireshark, nmap, and ping -t/ping -c.

9.FTP uses TCP at ports 21 and 20. It also provides authentication and directory operations, and it supports both active and passive modes. TFTP uses UDP at port 69, reads and writes files, has no authentication, no directory listings and small protocol. TFTP is not reliable on its own.

10.

a. 207.30.155.0: 207 is class C with a default mask of /24. There are 2^8-2 available hosts, or 254 usable hosts.

b. 123.56.0.0: 123 is a class A with a default mask of /8. There are 24^4-2 hosts, or 16,777,214 usable hosts.

c. 56.0.0.0: 56 is a class A, with a default mask of /8. There are 24^4-2 hosts, or 16,777,214 usable hosts.

11.

a. 201.210.59.87: Class C, the default mask is 255.255.255.0, with a network address of 201.210.59.0 and host portion of .87.

b. 56.112.87.191: Class A, the default mask is 255.0.0.0, with a network of 56.0.0.0 and a host portion of 112.87.191.

c. 112.158.2.104: Class A, the default mask is 255.0.0.0, with a network of 112.0.0.0 and a host portion of 158.2.104.

d. 192.158.75.89: Class C, the default mask is 255.255.255.0, with a network of 192.158.75.0 and a host portion of .89.

e. 228.195.128.241: Class E, reserved//experimental with no normal default mask and not usable for public host addressing.

12.

a. 127.0.0.1: Used for loopback to the local host.

b. 255.255.255.255: Used as a limited broadcast address.

c. 244.122.89.3: Class E that isn’t used for normal public host assignment.

d. 127.255.255.255: Still inside the loopback space, so it is used for loopback.

13.

a. 89.112.45.3: 01011001.01110000.00101101.00000011

b. 158.89.112.101: 10011110.01011001.01110000.01100101

c. 189.56.147.125: 10111101.00111000.10010011.0111101

d. 10.251.178.238: 00001010.11111011.10110010.11101110

B. Network Design Using IPv4 Addressing

- You would need to borrow enough host bits to create at least 8 network. Borrowing 3 bits would create 2^3, or 8 subnets. This will work because it is greater than the number needed. From there you must determine the new subnet mask. We have a Class B default mask of /16, and if we borrow 3 bits, we have a new subnet mask of /19. /19 in decimal is 255.255.224.0. Block size in the 3rd octet is 256-224 or 32. The subnet range would increase by 32 in the 3rd octet.

| City | Assigned Network | First Usable | Last Usable | Broadcast | Mask |
| --- | --- | --- | --- | --- | --- |
| Pensacola | 190.152.0.0/19 | 190.152.0.1 | 190.152.31.254 | 190.152.31.255 | 255.255.224.0 |
| Tallahassee | 190.152.32.0/19 | 190.152.32.1 | 190.152.63.254 | 190.152.63.255 | 255.255.224.0 |
| Jacksonville | 190.152.64.0/19 | 190.152.64.1 | 190.152.95.254 | 190.152.95.255 | 255.255.224.0 |
| Daytona Beach | 190.152.96.0/19 | 190.152.96.1 | 190.152.127.254 | 190.152.127.255 | 255.255.224.0 |
| Orlando | 190.152.128.0/19 | 190.152.128.1 | 190.152.159.254 | 190.152.159.255 | 255.255.224.0 |
| Tampa | 190.152.160.0/19 | 190.152.160.1 | 190.152.191.254 | 190.152.191.255 | 255.255.224.0 |
| Miami | 190.152.192.0/19 | 190.152.192.1 | 190.152.223.254 | 190.152.223.255 | 255.255.224.0 |

Subnet 8 would be unassigned and can be reserved for growth or future cities that will be included into the network.

C. IPv6 Addressing

1. Expand to full IPv6 addresses

a. 1522:8765:21::/48

1522:8765:0021:0000:0000:0000:0000:0000/48

b. 1789:1011:0::/64

1789:1011:0000:0000:0000:0000:0000:0000/64

c. FE80::0202:B3FF:FE1E:8329/56

FE80:0000:0000:0000:0202:B3FF:FE1E:8329/56

d. ::/128

0000:0000:0000:0000:0000:0000:0000:0000/128

e. 2002:64C8:64C8::/64

2002:64C8:64C8:0000:0000:0000:0000:0000/64

f. 2001:CDBA::3257:9652

2001:cdba:0000:0000:0000:0000:3257:9652

2. Compress to shortest IPv6 Form

a. 	FE80:0000:0000:0000:0202:B3FF:FE1E:8329

FE80::202:B3FF:FE1E:8329

b.	2001:0db8:85a3:0000:0000:8a2e:0370:7334

2001:DB8:85A3::8A2E:370:7334

c.	2041:0000:130F:0000:0000:07CO:853A:140B

2041:0:130F::7C0:853A:140B

d.	2001:cdba:0000:0000:0000:0000:3257:9652

2001:CDBA::3257:9652

e.	fe80:0:0:0:200:f8ff:fe21:67cf

FE80::200:F8FF:FE21:67CF

f.	2001:0000:3238:DFE1:0063:0000:0000:FEFB

2001:0:3238:DFE1:63::FEFB